### Steering Assembly – Assembly Steps

#### Step 1

Insert the Magnet and 1x M3 Nylock Nut into Motor Connector 1.

#### Step 2

Use 1x M3xØ4x6 Shoulder Bolt and 1x M3 Nylock Nut to attach Motor Connector 1 to Motor Connector 2.

#### Step 3

Use 1x M3xØ4x6 Shoulder Bolt and 1x M3 Nylock Nut to connect the assembly to the Steering Rod.

#### Step 4

Connect each Front Wheel Hub Assembly to the assembly using 1x M3xØ4x6 Shoulder Bolt and 1x M3 Nylock Nut for each.

#### Step 5

Ensure all connections are secure and rotate freely.

### Sensor Plate Assembly – Assembly Steps

#### Step 1

Place the Sensor Plate on your workspace. Insert 3x M3x10 Bolts into the Sensor Plate at the marked locations.

#### Step 2

Mount the RPLidar on the Sensor Plate by aligning the bolt holes. Install 4x M3x10 Bolts to secure the RPLidar to the Sensor Plate from the underside.

#### Step 3

Attach 3x M3x20 Standoff Nuts into the assembly at the marked positions.

#### Step 4

Carefully place the PCB and align the holes with the 3x M3x20 Standoff Nuts.

#### Step 5

Use 3x M3x10 Standoffs to firmly secure the PCB to the standoffs.

#### Step 6

Double-check that all Bolts and standoffs are properly tightened, and the assembly is secure.

Note: This assembly can be assembled from bottom to top on the Main Body if there is no intention of having the lidar wire pointing forwards.

### Rear Wheel Assembly – Assembly Steps

#### Step 1

Place 3x M3 Nylock Nuts into the designated recesses in the Rear Wheel Nut.

#### Step 2

Align the Tire with the Rear Wheel Hub and insert the assembly onto the Rear Wheel Nut.

#### Step 3

Insert 3x M3x25 Bolt through the Rear Wheel Hub and thread them into the M3 Nylock Nuts to secure the assembly.

#### Step 4

Double-check that the tire is seated evenly on the rim and that all fasteners are tightened securely.

Note: This is only for people using the 3dp wheel retention and not the metal hub.

### Camera Assembly – Assembly Steps

#### Step 1

Align the Camera to the Camera Mount so the holes are aligned.

#### Step 2

Insert 2x M4x10 Bolt through the holes in the Camera Mount and thread them into the Camera to secure it in place.

#### Step 3

Ensure that the Camera is firmly attached and flush with the Camera Mount.

### Front Wheel Hub Assembly – Assembly Steps

#### Step 1

Insert the M3 Nylock Nut and Front Wheel Stub into the Wheel Hub.

#### Step 2

Insert 2x 8mm ID x 12mm OD Bearings into the designated slots in the Front Wheel.

#### Step 3

Install the Tire onto the Front Wheel, ensuring it is fully seated around the rim.

#### Step 4

Fit the Wheel Assembly into the Hub Assembly.

#### Step 5

Secure the Wheel Assembly onto the assembly by using an M3x25 Bolt into the M3 Nylock Nut and Front Wheel Washer to ensure the wheel hub is fully assembled.

#### Step 6

Ensure the entire assembly is properly aligned, and the tire is centered on the hub for a secure fit.

### Cover Assembly – Assembly Steps

#### Step 1

Insert 2xM3x15 Bolts and 4x M3 Nylock Nuts from the bottom.

#### Step 2

Attach the AS5600 to the top using 4xM3x15 and the 4x M3 Nylock Nuts

#### Step 3

Install 2x M3x15 Standoffs into the designated holes on the Bottom Plate.

#### Step 4

Align the Front Bumper with the installed standoffs and holes in the Bottom Plate.

#### Step 5

Secure the Front Bumper to the assembly by threading 2x M3x15 Standoff Nuts into the previously installed standoffs.

### Main Body Assembly – Assembly Steps

Step 1
Place the Main Body on your workspace. Insert 8x M3 Nylock Nuts into the marked locations to prepare for secure mounting of the Cover Assembly.

Step 2
Align the 4x M2.5x10 Standoff Nuts and secure them using 4x M2.5x6 Bolts at the indicated positions.

Step 3
Mount the Stepper Motors into the Main Body by aligning the motor and securing it with 4x M3x6 Bolts.

Step 4
Install the Switch by pushing it into the slot.

#### Step 5

Install the Steering Motor using 2x M4 Nylock Nuts to hold it in place and 2x M4x30 Bolts to fasten securely.

Step 6
Mount the Jetson Nano by carefully aligning it into the standoffs and secure it using 4x M2.5x6 Bolts.

Step 7
Verify that all screws, standoffs, and modules are properly aligned and tightly secured to complete the Main Body Assembly.

### Car with Rear Hub Assembly – Assembly Steps

Step 1
Secure the Rear Bumper using 2x M3x25 Bolts and 2x M3 Nylock Nuts into the Main Body Assembly at the designated locations.

Step 2
Mount the Rear Wheel Hubs onto the Stepper Motor Shafts and secure it with the included set screws.

Step 3
Attach the Rear Wheel to the Rear Wheel Hubs using 6x M3x15 Bolts.

Step 4

Attach the Sensor Plate Assembly and secure it using 2x M3x10 Standoffs into the Main Body Assembly, 2x M3x6 into those standoffs through the PCB, and 3x M3x10 Bolts through the Main Body into the Sensor Plate Standoffs.

Step 5
Attach the Steering Assembly and secure it to the Steering Motor using M3x10 going through the Motor Connector into the M3 Nylock Nuts that was placed there. Use 2x M3xØ4x6 to attach the wheel hubs to the vehicle.

Step 6
Mount the Cover Assembly using 2x M3xØ4x6, 2x M3x15 and 8x M3x10 Bolts to ensure it is tightly attached to the Main Body Assembly.

Step 7

Mount the Camera Mount Assembly using 4x M3x15 in the indicated positions.

Step 8

Slide the Battery Cage in.

Step 9

Clip the Shrouds on.

Step 10

Verify that all screws are firmly tightened, and the assembly is complete.